Lab Practical #02:

Study of different network devices in detail.

Practical Assignment #02:

- Give difference between below network devices.

- Hub and Switch

- Switch and Router

- Router and Gateway

- Working of below network devices:

  - Repeater

  - Modem((DSL and ADSL)

  - Hub

  - Bridge

  - Switch

  - Router

  - Gateway

# Hub and Switch

| No. | Hub | Switch |
| --- | --- | --- |
| 1 | Broadcasts data to all connected devices | Forwards data to the specific device (MAC) |
| 2 | Works at Physical Layer (Layer1) | Works at Data Link Layer (Layer 2) |
| 3 | Non-selective – sends data to everyone | Selective – sends data only to the target |
| 4 | Low – causes unnecessary traffic | High – reduces network traffic |
| 5 | Single collision domain for all ports | Each port has a separate collision domain |
| 6 | Does not store or use MAC addresses | Maintains a table of MAC addresses |
| 7 | Cheaper than a switch | More expensive than a hub |

# Switch and Router

| No. | Switch | Router |
| --- | --- | --- |
| 1 | Connects devices within a local network (LAN) | Connects different networks (e.g., LAN to WAN) |
| 2 | Works at Data Link Layer (Layer 2) | Works at Network Layer (Layer 3) |
| 3 | Uses MAC address to forward data | Uses IP address to route data |
| 4 | Sends data to the correct device in the same network | Sends data between networks like LAN to internet |
| 5 | Smart – forwards data to specific devices | Smarter – decides the best path for data |
| 6 | Network bridge (within one network) | Network gateway (between networks) |
| 7 | Connecting computers, printers in an office | Connecting your home network to the internet |

# Router and Gateway

| No. | Router | Gateway |
| --- | --- | --- |
| 1 | Forwards data between similar networks (same protocols).. | Connects different networks by translating communication protocols.. |
| 2 | Operates mainly at Network Layer (Layer 3).. | Can operate at any OSI layer, depending on implementation.. |
| 3 | To route data packets across networks with the same protocol (like IP)... | To enable communication between networks using different protocols.. |
| 4 | To route data packets across networks with the same protocol (like IP).. | Translates, reformats, and forwards data between incompatible networks.. |
| 5 | Commonly used in LAN-to-LAN or LAN-to-WAN scenarios.. | Used to connect enterprise networks to the Internet or other protocol systems.. |
| 6 | Breaks up collision domains; each port/interface is its own collision domain.. | Does not break collision domains; not designed for that purpose. |

# Working of below network devices:

### 1. Repeater

- Definition: A repeater is a device that regenerates and amplifies signals in a network.

- Working:
It works at the physical layer (Layer 1) of the OSI model. When a signal travels a long distance, it becomes weak or distorted. The repeater receives the weak signal, cleans it from noise, amplifies it, and then transmits it again at full strength.

- Use: Used to extend the range of a network by boosting signal strength across long distances.

2. Modem (DSL and ADSL)

- Definition: A Modem (MOdulator-DEModulator) is used to connect a computer or network to the Internet over telephone lines.

- Working:
It converts digital signals from a computer into analog signals for transmission (modulation) and then converts received analog signals back into digital (demodulation).

  - DSL (Digital Subscriber Line): Provides symmetric speeds (same upload and download).

  - ADSL (Asymmetric DSL): Provides higher download speed and lower upload speed, ideal for home use.

- Use: Acts as a bridge between the Internet Service Provider and the user’s computer or router.

3. Hub

- Definition: A hub is a basic network device that connects multiple computers in a LAN.

- Working:
Works at the physical layer. It receives data from one port and broadcasts it to all other ports, regardless of the destination. It does not check who the data is for.

- Use: Simple but inefficient; used in small or temporary networks.

4. Bridge

- Definition: A bridge connects two or more LANs and manages data flow between them.

- Working:
Works at the data link layer (Layer 2). It checks the MAC address of incoming data and decides whether to forward or block it. This helps reduce unnecessary traffic and manage bandwidth efficiently.

- Use: Useful in dividing large networks and improving performance.

5. Switch

- Definition: A switch is an intelligent device that connects devices in a LAN and forwards data only to the intended recipient.

- Working:
Operates at the data link layer. It uses a MAC address table to identify devices and sends data only to the correct port, unlike a hub. This improves speed and reduces congestion.

- Use: Commonly used in all modern local area networks (LANs).

6. Router

- Definition: A router connects different networks and manages data traffic between them.

- Working:
Works at the network layer (Layer 3). It reads the IP address of each data packet and uses a routing table to find the best path to the destination. Routers can also assign IP addresses and provide security features like firewalls.

- Use: Used to connect home or office networks to the Internet.

7. Gateway

- Definition: A gateway is a device that connects two different networks using different protocols.

- Working:
A gateway can work at any layer of the OSI model. It acts as a translator, converting data from one protocol format to another so that devices in different networks (e.g., TCP/IP and IBM mainframe) can communicate.

- Use: Used in enterprises to connect internal networks to external ones like the Internet.